CCP-CNS2026 — Computer Networks

Problem 8: Hybrid Cloud

Week 4 Status Report — Advanced Features & Final Integration

Date: June 10, 2026  |  GitHub: github.com/anastahir00/CCP-HybridCloud

# 1. Week 4 Overview

This report documents all Week 4 deliverables for Problem 8: Hybrid Cloud. The focus this week was Advanced Features and Final Integration, including NAT configuration, QoS policy implementation, performance baseline testing, backup automation, and final GNS3 project export.

All tasks were completed by Anas (Network Architect) due to team member unavailability. Known platform limitations (Hyper-V virtual switch blocking cross-network traffic) have been documented in detail.

# 2. Team Contributions — Week 4

| Role | Assigned To | Actual Contributor | Tasks Completed |
| --- | --- | --- | --- |
| Network Architect | Anas | Anas ✅ | NAT, QoS, GNS3 export, configs |
| Security Engineer | Talha | No contribution ❌ | — |
| Testing / QA Engineer | Saim | Anas ✅ | Ping tests, test matrix, performance baseline |
| Automation Lead | Nihaas | Nihaas ✅ | backup_config.py script (GitHub) |
| Documentation Lead | Salik | Anas ✅ | Week 4 status report, screenshots |

# 3. NAT Configuration

## 3.1 Objective

Configure NAT overload (PAT) on the Cisco on-premise gateway router to allow internal LAN hosts (172.16.1.0/24) to communicate through the WAN interface (FastEthernet2/0, IP: 10.0.0.1).

## 3.2 Configuration Applied

interface FastEthernet1/0

ip nat inside

!

interface FastEthernet2/0

ip nat outside

!

ip nat inside source list 1 interface FastEthernet2/0 overload

access-list 1 permit 172.16.1.0 0.0.0.255

## 3.3 Verification

NAT statistics confirmed correct interface assignments:

- Outside interface: FastEthernet2/0 (10.0.0.1)

- Inside interface: FastEthernet1/0 (172.16.1.1)

- Dynamic mapping: access-list 1 → FastEthernet2/0 overload

Note on NAT translation hits:

NAT translations showed 0 hits because end-to-end traffic between the GNS3 network and the Docker cloud container is blocked by the Hyper-V internal virtual switch (a known platform limitation documented since Week 3). NAT configuration has been verified structurally via show ip nat statistics.

# 4. QoS / Rate Limiting Configuration

## 4.1 Objective

Implement a QoS policy on the LAN interface to prioritize critical traffic (DSCP EF / Expedited Forwarding) at 30% guaranteed bandwidth, with fair-queuing for all other traffic.

## 4.2 Configuration Applied

class-map match-all CRITICAL

match ip dscp ef

!

policy-map QOS-POLICY

class CRITICAL

priority percent 30

class class-default

fair-queue

!

interface FastEthernet1/0

service-policy output QOS-POLICY

## 4.3 Verification — show policy-map interface FastEthernet1/0

Output confirmed:

- Service-policy output: QOS-POLICY applied on FastEthernet1/0

- Class CRITICAL: DSCP EF (46), Priority 30% = 30,000 kbps, burst 750,000 bytes

- Class class-default: Fair-queue, per-flow queue limit 16 packets, 4 packets already processed

# 5. Performance Baseline Testing

## 5.1 Methodology

iPerf3 could not be installed on the Ubuntu VM due to the absence of internet connectivity (Ubuntu apt repositories are unreachable from the isolated GNS3 network). Performance baseline was measured using ICMP ping between the Ubuntu on-premise server (172.16.1.10) and the Cisco router LAN interface (172.16.1.1).

## 5.2 Latency Baseline Results

| Metric | Value | Notes |
| --- | --- | --- |
| Source | 172.16.1.10 (Ubuntu VM) | On-premise server |
| Destination | 172.16.1.1 (Cisco Router) | LAN gateway |
| Packets Sent | 5 | ping -c 5 |
| Packet Loss | 0% | ✅ PASS |
| Min RTT | 6.76 ms | — |
| Avg RTT | 18.13 ms | — |
| Max RTT | 50.14 ms | — |
| Mdev | 16.27 ms | Jitter |

# 6. Backup Automation Script

## 6.1 Script: backup_config.py

Developed by: Nihaas (Automation Lead)

import paramiko

import datetime

import os

router_ip = '10.0.0.1'

username  = 'cisco'

password  = 'cisco'

os.makedirs('backups', exist_ok=True)

try:

ssh = paramiko.SSHClient()

ssh.set_missing_host_key_policy(paramiko.AutoAddPolicy())

ssh.connect(router_ip, username=username, password=password, timeout=10)

stdin, stdout, stderr = ssh.exec_command('show running-config')

config = stdout.read().decode()

timestamp = datetime.datetime.now().strftime('%Y%m%d_%H%M%S')

filename  = f'backups/R1_backup_{timestamp}.txt'

with open(filename, 'w') as f:

f.write(config)

print(f'[SUCCESS] Backup saved to {filename}')

ssh.close()

except Exception as e:

print(f'[ERROR] Could not back up router: {e}')

## 6.2 Features

- Connects to Cisco router via SSH using Paramiko

- Executes 'show running-config' and saves output to timestamped file

- Creates backups/ directory automatically if it does not exist

- Includes error handling: prints error message if router is unreachable

- File naming convention: R1_backup_YYYYMMDD_HHMMSS.txt

# 7. Final Test Matrix

| ID | Test Description | Expected Result | Actual Result | Status |
| --- | --- | --- | --- | --- |
| T01 | Ubuntu VM → Router ping (172.16.1.1) | 0% loss | 0% loss, avg 18ms | ✅ PASS |
| T02 | Ubuntu VM → Docker ping (10.0.0.10) | 0% loss | 100% loss (Hyper-V) | ⚠️ Partial |
| T03 | NAT inside/outside interfaces assigned | Fa1/0 inside, Fa2/0 outside | Confirmed via show ip nat statistics | ✅ PASS |
| T04 | QoS policy active on FastEthernet1/0 | QOS-POLICY applied | CRITICAL + class-default active | ✅ PASS |
| T05 | CRITICAL class DSCP EF priority | 30% bandwidth guaranteed | 30,000 kbps priority confirmed | ✅ PASS |
| T06 | IPsec SA present on router | SA entries present | Configured (ESP blocked by Hyper-V) | ⚠️ Partial |
| T07 | Backup script executes without error | Config file saved | Script functional (SSH to router) | ✅ PASS |
| T08 | iPerf3 throughput baseline | >10 Mbps | N/A — apt repos unreachable | ⚠️ N/A |

# 8. Known Limitations & Platform Constraints

| Issue | Root Cause | Mitigation |
| --- | --- | --- |
| Ubuntu → Docker ping fails | Hyper-V internal switch isolates GNS3 cloud node from Docker network | Documented; both sides configured correctly |
| NAT translations = 0 hits | No cross-network traffic due to Hyper-V | NAT verified via interface assignment |
| QoS counters = 0 packets | No significant traffic generated | Policy application confirmed via show command |
| iPerf3 not installable | Ubuntu VM has no apt repo access | ICMP ping used for latency baseline |
| ESP packets not visible in Wireshark | Hyper-V virtual switch strips or blocks ESP | Documented in Week 3 report |

# 9. Files Pushed to GitHub This Week

| File Path | Purpose |
| --- | --- |
| docs/week4/configs/router-nat-config.txt | NAT configuration commands |
| docs/week4/configs/router-qos-config.txt | QoS policy configuration |
| docs/week4/configs/router-ipsec-acl.txt | IPsec ACL (Week 3 carry-over) |
| docs/week4/screenshots/nat-statistics.png | NAT statistics verification |
| docs/week4/screenshots/qos-verified.png | QoS policy active on Fa1/0 |
| docs/week4/screenshots/ubuntu-router-ping.png | LAN ping success (performance baseline) |
| docs/week4/screenshots/ubuntu-docker-ping-fail.png | Cross-network failure (documented) |
| docs/week4/iperf-baseline.txt | Performance baseline (ping data) |
| docs/week4/test-matrix.md | Complete pass/fail test results |
| docs/week4/gns3-project/HybridCloud.zip | Final GNS3 portable project export |
| docs/weekly-status/week4.md | Week 4 status report (Markdown) |
| scripts/backup_config.py | Router backup automation script |

# 10. Week 4 Summary & Week 5 Plan

## 10.1 Week 4 Outcome

| Metric | Status |
| --- | --- |
| All deliverables completed | ✅ Complete |
| Configurations syntactically correct | ✅ Verified |
| End-to-end verification | ⚠️ Partial (Hyper-V) |
| Documentation complete | ✅ Complete |
| GNS3 project exported | ✅ Uploaded |

## 10.2 Week 5 Plan

- Final report compilation (15-20 pages) covering all weeks

- Video demonstration (8-10 min, narrated, YouTube unlisted)

- Team presentation slides and 12-minute recording

- Incident response report (3-5 pages) — IPsec tunnel attack scenario

- Peer evaluation forms completed individually by all members