WEEK 3 IPSEC DOCUMENTATION

Problem 8: Hybrid Cloud  |  CCP-CNS2026  |  Computer Networks

Week 3 of 5  |  Date: June 10, 2026  |  IPsec VPN Configuration

| Overall Status | Router IPsec | StrongSwan | Tunnel Status |
| --- | --- | --- | --- |
| 🟡 PARTIAL | ✅ Configured | ✅ Running | ⚠ Blocked |

# 1. Overview

This document covers the Week 3 IPsec tunnel configuration for the Hybrid Cloud project. The objective was to establish an encrypted IPsec VPN tunnel between the on-premise Cisco router (GNS3) and the cloud-side StrongSwan gateway (Docker container). Both sides were fully configured; however, the tunnel could not complete IKE Phase 1 due to a Windows Hyper-V networking limitation described in Section 6.

# 2. Team Members & Roles

| Role | Name | ID | Week 3 Tasks |
| --- | --- | --- | --- |
| Project Lead + Network Architect | Muhammad Anas Tahir | F2024266985 | Router IPsec + StrongSwan + Testing |
| Security Engineer | Muhammad Talha Bhatti | F2024266503 | (Handled by Anas) |
| Automation Lead | Muhammad Nihaas Bhatti | F2024266196 | Scripts (Week 4) |
| Testing/QA Engineer | Saim Ullah | F2024266594 | (Handled by Anas) |
| Documentation Lead | Salik Saeed | F2024266596 | (Handled by Anas) |

# 3. IPsec Tunnel Design

## 3.1 Network Architecture

| Component | Device | IP Address | Role |
| --- | --- | --- | --- |
| On-Premise Gateway | Cisco Router (GNS3) | 10.0.0.1 / 172.16.1.1 | IPsec Initiator |
| On-Premise Server | Ubuntu Server (VirtualBox) | 172.16.1.10 | LAN Host |
| Cloud Gateway | StrongSwan (Docker) | 10.0.0.10 | IPsec Responder |
| IPsec Tunnel | GNS3 Fa2/0 ↔ Docker eth0 | 10.0.0.1 ↔ 10.0.0.10 | Encrypted Channel |

## 3.2 IPsec Parameters

| Parameter | Value | Notes |
| --- | --- | --- |
| IKE Version | IKEv1 | Cisco c7200 uses IKEv1 Main Mode by default |
| Phase 1 Encryption | AES-256 | Strong encryption, both sides configured |
| Phase 1 Hash | SHA-1 | Matched between Cisco and StrongSwan |
| DH Group | Group 2 (1024-bit) | Compatible with Cisco c7200 |
| Phase 2 Encryption | AES-256 | ESP transform set |
| Phase 2 Hash | SHA-1 | ESP-AES-SHA transform set |
| Authentication | Pre-Shared Key (PSK) | PSK: HybridCloud2026 |
| Protected Traffic | 172.16.1.0/24 ↔ 10.0.0.0/24 | ACL 100 on router, leftsubnet/rightsubnet on StrongSwan |
| Tunnel Mode | ESP Tunnel | Full IP packet encapsulation |

# 4. Cisco Router IPsec Configuration

## 4.1 ISAKMP Policy (Phase 1)

crypto isakmp policy 10

encryption aes 256

hash sha

authentication pre-share

group 2

lifetime 86400

exit

## 4.2 Pre-Shared Key

crypto isakmp key HybridCloud2026 address 10.0.0.10

## 4.3 IPsec Transform Set (Phase 2)

crypto ipsec transform-set ESP-AES-SHA esp-aes 256 esp-sha-hmac

mode tunnel

exit

## 4.4 Crypto Map

crypto map CMAP 10 ipsec-isakmp

set peer 10.0.0.10

set transform-set ESP-AES-SHA

match address 100

exit

## 4.5 Access List (Traffic to Encrypt)

access-list 100 permit ip 172.16.1.0 0.0.0.255 10.0.0.0 0.0.0.255

## 4.6 Apply to Interface

interface FastEthernet2/0

crypto map CMAP

exit

write memory

# 5. StrongSwan Configuration (Cloud Side)

## 5.1 Installation

StrongSwan 5.9.5 was installed inside the Docker container (cloud-server) running Ubuntu 22.04 with --privileged mode to allow kernel IPsec operations:

docker run -d --name cloud-server --network hybridnet --ip 10.0.0.10 --privileged ubuntu:22.04 sleep infinity

docker exec -it cloud-server bash

apt-get update && apt-get install -y strongswan

## 5.2 /etc/ipsec.conf

config setup

charondebug="all"

uniqueids=no

conn hybrid-cloud

left=10.0.0.10

leftsubnet=10.0.0.0/24

right=10.0.0.1

rightsubnet=172.16.1.0/24

authby=secret

keyexchange=ikev1

ike=aes256-sha1-modp1024

esp=aes256-sha1

type=tunnel

auto=start

forceencaps=yes

## 5.3 /etc/ipsec.secrets

10.0.0.10 10.0.0.1 : PSK "HybridCloud2026"

## 5.4 Starting StrongSwan

ipsec start

ipsec statusall

# 6. Verification Results

| Test | Command | Result | Notes |
| --- | --- | --- | --- |
| Router ISAKMP SA | show crypto isakmp sa | MM_NO_STATE | IKE initiated, no response from peer |
| Router IPsec SA | show crypto ipsec sa | 0 pkts encrypt | Tunnel not established yet |
| StrongSwan Status | ipsec statusall | CONNECTING | Attempting IKEv1 Main Mode |
| StrongSwan Listening | ss -ulnp ¦ grep 500 | ✅ Listening | UDP 500 and 4500 open |
| Wireshark ISAKMP | Filter: isakmp ¦¦ esp | No packets | Hyper-V blocking UDP 500 |
| Container → R1 Ping | ping 10.0.0.1 -c 4 | ✅ 0% loss | ICMP works one direction |

# 7. Platform Limitation & Root Cause Analysis

## 7.1 Problem Statement

The IPsec tunnel could not fully establish due to a Windows Hyper-V Internal Switch networking limitation. Both sides are correctly configured but IKE packets (UDP 500/4500) cannot traverse between the GNS3 Cloud node and the Docker container.

## 7.2 Technical Root Cause

- Docker Desktop on Windows uses Hyper-V virtual switches of type 'Internal'

- Hyper-V Internal switches only allow communication between the Windows host and VMs — not between external virtual machines (like GNS3)

- GNS3's Cloud node bridges to the vEthernet (WSL) adapter, but this adapter is on the Hyper-V Internal switch

- IKE packets from GNS3 router (10.0.0.1) to Docker container (10.0.0.10) are dropped by the Hyper-V layer

- External VMSwitch (which would allow bridging) is not supported in this environment

## 7.3 Evidence

- R1 debug shows: 'sending packet to 10.0.0.10 my_port 500' — packets sent but no response

- StrongSwan shows: 'sending retransmit 5 of request, giving up' — same issue from cloud side

- ARP table shows 10.0.0.2 as 'Incomplete' — Windows host not responding to ARP from GNS3

- Wireshark on vEthernet (WSL) shows no ISAKMP or ESP packets

## 7.4 Resolution Plan

- Week 4: Attempt tunnel establishment using GNS3 VM as Docker host instead of Windows host

- Alternative: Use Linux VM to run Docker natively for full kernel IPsec support

- Both sides remain correctly configured — tunnel will work on a Linux-based host

# 8. Week 3 Deliverables

| Deliverable | Status | Location |
| --- | --- | --- |
| Router IPsec configuration | ✅ Done | docs/week3/configs/router-crypto-config.txt |
| show crypto isakmp sa screenshot | ✅ Done | docs/week3/screenshots/router-isakmp-sa.png |
| show crypto ipsec sa screenshot | ✅ Done | docs/week3/screenshots/router-ipsec-sa.png |
| StrongSwan ipsec.conf | ✅ Done | docs/week3/configs/ipsec.conf |
| StrongSwan ipsec.secrets | ✅ Done | docs/week3/configs/ipsec.secrets |
| ipsec statusall screenshot | ✅ Done | docs/week3/screenshots/strongswan-status.png |
| Wireshark PCAP capture | ✅ Done | docs/week3/week3-esp-capture.pcap |
| Week 3 status report | ✅ Done | docs/weekly-status/week3.md |
| Platform limitation documented | ✅ Done | This document + week3.md |
| Full tunnel established | ⚠ Blocked | Hyper-V limitation — Week 4 resolution planned |

# 9. Week 4 Plan

- NAT configuration on router

- QoS / rate limiting implementation

- Performance baseline testing with iPerf

- Automation scripts (deploy_ipsec.py, backup_config.py)

- Attempt tunnel resolution via Linux-based Docker host

- Final GNS3 project v3 with all configs

Prepared by: Muhammad Anas Tahir (Project Lead)  |  CCP-CNS2026  |  Problem 8: Hybrid Cloud